/*快速中断*/

//GPIO_2的CH1对应的button和定时器TIMER_0是中断源

#include "xil_io.h"

#include "stdio.h"

#include "xgpio_l.h"

#include "xintc_l.h"

#include "xtmrctr_l.h"

#include "xio.h"

#include "xil_exception.h"

#define RESET_VALUE 100000

void Seg_TimerCounterHandler()__attribute__ ((fast_interrupt));

//快速中断服务程序Seg_TimerCounterHandler()

void BTN_sw_seg()__attribute__ ((fast_interrupt));

//快速中断服务程序BTN_sw_seg()

//二者都是可以单独响应的

short segcode[8]={0xff,0xff,0xff,0xff,0xff,0xff,0xff,0xff};//段码显示缓冲区，初始默认全部显示为灭，根据所选模式将segtable内的数值写入对应的位置

char segtable[16]={0xc0,0xf9,0xa4,0xb0,0x99,0x92,0x82,0xf8,0x80,0x90,0x88,0x83,0xc6,0xa1,0x86,0x8e};//段码表 16进制数字 [0-f] 的16进制码

//char scancode[5][2] = {{0x1,0xc6},{0x2,0xc1},{0x4,0xc7},{0x8,0x88},{0x10,0xa1}};//16进制 按键 对应显示

short pos=0xff7f;//位码，用于扫描

int i=0;//用于扫描

int main()

{

/*GPIO*/

/*GPIO_0*/

//GPIO:SW15~0  GPIO2:LED15~0

Xil_Out16(XPAR_AXI_GPIO_0_BASEADDR+XGPIO_TRI_OFFSET,0xffff);//设定16个开关为输入

/*GPIO_1*/

//GPIO:AN7~0  GPIO2:DP~CA

Xil_Out8(XPAR_AXI_GPIO_1_BASEADDR+XGPIO_TRI_OFFSET,0x0);//设定位码为输出

Xil_Out8(XPAR_AXI_GPIO_1_BASEADDR+XGPIO_TRI2_OFFSET,0x0);//设定段码为输出

/*GPIO_2*/

//GPIO:BTND,U,L,R,C  GPIO2:LD17(B\G\R),LD16(B\G\R)

Xil_Out8(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_TRI_OFFSET,0x1f);//设定5个按键为输入

/*此处的中断触发是按键，如果是sw之类的那就改为GPIO0之类的中断输出*/

Xil_Out32(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_IER_OFFSET,XGPIO_IR_CH1_MASK);//CH1允许 中断，CH1对应的是按键，CH2是LD

Xil_Out32(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_GIE_OFFSET,XGPIO_GIE_GINTR_ENABLE_MASK);//允许 按键和LD所对应的GPIO IP核 GPIO_2 中断输出

/*TIMER*/

/*TIMER_0*/

Xil_Out32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET,

Xil_In32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET)&~XTC_CSR_ENABLE_TMR_MASK);//停止计数器，写TCSR，清除中断

Xil_Out32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TLR_OFFSET,RESET_VALUE);//写TLR，预置计数初值

Xil_Out32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET,

Xil_In32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET)|XTC_CSR_LOAD_MASK);//获取计数初值

Xil_Out32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET,(Xil_In32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET)&~XTC_CSR_LOAD_MASK)\

|XTC_CSR_ENABLE_TMR_MASK|XTC_CSR_AUTO_RELOAD_MASK|XTC_CSR_ENABLE_INT_MASK|XTC_CSR_DOWN_COUNT_MASK);

//开始计数运行，自动获取，允许中断，减计数

/*INTC*/

//快速中断在中断服务程序中不需要写程序手动清IAR

Xil_Out32(XPAR_AXI_INTC_0_BASEADDR+XIN_IAR_OFFSET,0x6);

//初始化清除中断状态，写IAR寄存器

Xil_Out32(XPAR_AXI_INTC_0_BASEADDR+XIN_IMR_OFFSET,0x6);//0x0110 使得D3,D2为1

//写中断模式寄存器IMR，使中断控制器工作在快速中断模式，普通中断则不需要这一步

Xil_Out32(XPAR_AXI_INTC_0_BASEADDR+XIN_IER_OFFSET,XPAR_AXI_GPIO_2_IP2INTC_IRPT_MASK|XPAR_AXI_TIMER_0_INTERRUPT_MASK);

//写IER，开放中断控制器INTC对应的各个中断源输入引脚的中断，GPIO_2和TIMER_0是中断源，此处使能GPIO_2或TIMER_0所对应的输入引脚

Xil_Out32(XPAR_AXI_INTC_0_BASEADDR+XIN_MER_OFFSET,XIN_INT_MASTER_ENABLE_MASK|XIN_INT_HARDWARE_ENABLE_MASK);

//写MER，开放中断控制器输出引脚的中断

Xil_Out32(XPAR_AXI_INTC_0_BASEADDR+XIN_IVAR_OFFSET+0x4,(int)BTN_sw_seg);//偏移4

Xil_Out32(XPAR_AXI_INTC_0_BASEADDR+XIN_IVAR_OFFSET+0x8,(int)Seg_TimerCounterHandler);//偏移8

//填写中断控制器维护的中断向量表 写IVAR寄存器，就是把这两个快速中断服务程序的地址填到中断向量表的指定存储位置

/*MicroBlaze*/

microblaze_enable_interrupts();

//microblaze中断开放

return 0;

}

//快速中断直接跳转到中断服务程序，而不用跳转到0x10读取跳转地址

//中断源为定时器

void Seg_TimerCounterHandler()

{

Xil_Out8(XPAR_AXI_GPIO_1_BASEADDR+XGPIO_DATA_OFFSET,pos);//输出选择位码

Xil_Out8(XPAR_AXI_GPIO_1_BASEADDR+XGPIO_DATA2_OFFSET,segcode[i]);//输出此时i对应的段码

pos=pos>>1;//右移一位，从左到右扫描

i++;

if(i==8)//重置pos和i分别 为初始位码 以及 段码的起始位

{

i=0;

pos=0xff7f;

}

Xil_Out32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET,Xil_In32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET));

//将TCSR寄存器原来所存的值再次写入到TCSR寄存器中去，清除定时器的中断状态

}

//快速中断直接跳转到中断服务程序，而不用跳转到0x10读取跳转地址

//中断源为按钮

void BTN_sw_seg()

{

char button=Xil_In8(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_DATA_OFFSET);//读取按键键值

short cur_sw=Xil_In16(XPAR_AXI_GPIO_0_BASEADDR+XGPIO_DATA_OFFSET);//读取开关数值

if(button==0x01)//BTNC

{

for(int i=0;i<8;i++){

for(int n=0;n<8;n++){

segcode[7-n]=segtable[(cur_sw>>n)&0x1];

}

}

}

else if(button==0x02)//BTNU

{

for(int i=0;i<4;i++){

for(int n=0;n<4;n++){

segcode[7-n]=segtable[(cur_sw>>(4*n))&0xf];

}

for(int n=4;n<8;n++){

segcode[7-n]=0xff;

}

}

}

else if((button==0x10))//BTND

{

int k=cur_sw;//不改变cur_sw

for(int j=0;j<5;j++){

switch(k%10)

{

case(0):segcode[7-j]=segtable[0];break;

case(1):segcode[7-j]=segtable[1];break;

case(2):segcode[7-j]=segtable[2];break;

case(3):segcode[7-j]=segtable[3];break;

case(4):segcode[7-j]=segtable[4];break;

case(5):segcode[7-j]=segtable[5];break;

case(6):segcode[7-j]=segtable[6];break;

case(7):segcode[7-j]=segtable[7];break;

case(8):segcode[7-j]=segtable[8];break;

case(9):segcode[7-j]=segtable[9];break;

}

k/=10;

}

for(int n=5;n<8;n++){

segcode[7-n]=0xff;

}

}

Xil_Out32(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_ISR_OFFSET,Xil_In32(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_ISR_OFFSET));//写ISR寄存器存的的1(申请中断状态)到ISR寄存器中去，清除GPIO_2的申请中断状态

//因为是快速中断，INTC的中断请求状态不需要程序清除(返回的ack信号为01就给他清除了)，只需要清除中断源GPIO_2的中断状态

}