/*普通中断*/

//GPIO_2的CH1对应的button和定时器IMER_0是中断源

#include "xil_io.h"

#include "stdio.h"

#include "xgpio_l.h"

#include "xintc_l.h"

#include "xtmrctr_l.h"

#define RESET_VALUE 100000

void Seg_TimerCounterHandler();

void BTN_sw_seg();

//普通中断服务程序的子程序

void My_ISR() __attribute__ ((interrupt_handler));

//总的 普通中断服务程序

short segcode[8]={0xff,0xff,0xff,0xff,0xff,0xff,0xff,0xff};///段码显示缓冲区，初始默认全部显示为灭，根据所选模式将segtable内的数值写入对应的位置

char segtable[16]={0xc0,0xf9,0xa4,0xb0,0x99,0x92,0x82,0xf8,0x80,0x90,0x88,0x83,0xc6,0xa1,0x86,0x8e};//段选表 16进制数字 [0-f] 的16进制码

//char scancode[5][2] = {{0x1,0xc6},{0x2,0xc1},{0x4,0xc7},{0x8,0x88},{0x10,0xa1}};//16进制 按键 对应显示

short pos=0xff7f;//位选，用于扫描

int i=0;//用于扫描

int main()

{

/*GPIO*/

/*GPIO_0*/

//GPIO:SW15~0  GPIO2:LED15~0

Xil_Out16(XPAR_AXI_GPIO_0_BASEADDR+XGPIO_TRI_OFFSET,0xffff);//设定16个开关为输入

/*GPIO_1*/

//GPIO:AN7~0  GPIO2:DP~CA

Xil_Out8(XPAR_AXI_GPIO_1_BASEADDR+XGPIO_TRI_OFFSET,0x0);//设定位码为输出

Xil_Out8(XPAR_AXI_GPIO_1_BASEADDR+XGPIO_TRI2_OFFSET,0x0);//设定段码为输出

/*GPIO_2*/

//GPIO:BTND,U,L,R,C  GPIO2:LD17(B\G\R),LD16(B\G\R)

Xil_Out8(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_TRI_OFFSET,0x1f);//设定5个按键为输入

/*此处的中断触发是按键，如果是sw之类的那就改为GPIO_0之类的中断输出*/

Xil_Out32(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_IER_OFFSET,XGPIO_IR_CH1_MASK);//CH1允许 中断，CH1对应的是按键，CH2是LD

Xil_Out32(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_GIE_OFFSET,XGPIO_GIE_GINTR_ENABLE_MASK);//允许 按键和LD所对应的GPIO IP核 GPIO_2 中断输出

/*TIMER*/

Xil_Out32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET,

Xil_In32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET)&~XTC_CSR_ENABLE_TMR_MASK);//停止计数器，写TCSR，清除中断

Xil_Out32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TLR_OFFSET,RESET_VALUE);//写TLR，预置计数初值

Xil_Out32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET,

Xil_In32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET)|XTC_CSR_LOAD_MASK);//获取计数初值

Xil_Out32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET,(Xil_In32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET)&~XTC_CSR_LOAD_MASK)\

|XTC_CSR_ENABLE_TMR_MASK|XTC_CSR_AUTO_RELOAD_MASK|XTC_CSR_ENABLE_INT_MASK|XTC_CSR_DOWN_COUNT_MASK);

//开始计数运行，自动获取，允许中断，减计数

/*INTC*/

//普通中断在中断服务程序中要写程序手动清IAR

Xil_Out32(XPAR_AXI_INTC_0_BASEADDR+XIN_IAR_OFFSET,0x6);//不清理也没啥大问题

//初始化清除中断状态，写IAR寄存器

//不需要写中断模式寄存器IMR

Xil_Out32(XPAR_AXI_INTC_0_BASEADDR+XIN_IER_OFFSET,XPAR_AXI_GPIO_2_IP2INTC_IRPT_MASK|XPAR_AXI_TIMER_0_INTERRUPT_MASK);

//写IER，开放中断控制器INTC对应的各个中断源输入引脚的中断，GPIO_2和TIMER_0是中断源，此处使能GPIO_2或TIMER_0所对应的输入引脚

Xil_Out32(XPAR_AXI_INTC_0_BASEADDR+XIN_MER_OFFSET,XIN_INT_MASTER_ENABLE_MASK|XIN_INT_HARDWARE_ENABLE_MASK);

//写MER，开放中断控制器输出引脚的中断

//无需填写中断控制器维护的中断向量表，跳转至中断向量表内的0x10处即可

/*MicroBlaze*/

microblaze_enable_interrupts();

//microblaze中断开放

return 0;

}

//在中断向量表内的0x10处的imm和brai分别记录了My_ISR()地址的高、低16位

void My_ISR()

{

int status;

status=Xil_In32(XPAR_AXI_INTC_0_BASEADDR+XIN_ISR_OFFSET);//读取INTC的中断状态寄存器ISR

if((status&XPAR_AXI_TIMER_0_INTERRUPT_MASK)==XPAR_AXI_TIMER_0_INTERRUPT_MASK)//查询是否为TIMER产生的中断

{

Seg_TimerCounterHandler();//是就调用计时器的中断服务程序，即扫描输出相应位码以及所对应的7段数码管上应显示的数字的16位进制数

}

else if((status&XPAR_AXI_GPIO_2_IP2INTC_IRPT_MASK)==XPAR_AXI_GPIO_2_IP2INTC_IRPT_MASK)//查询是否为GPIO_2产生的中断

{

BTN_sw_seg();

}

Xil_Out32(XPAR_AXI_INTC_0_BASEADDR+XIN_IAR_OFFSET,status);

//普通中断，INTC需要将现在的中断状态ISR写到IAR寄存器中去，清除中断状态，而快速中断不用

}

//中断源为TIMER

void Seg_TimerCounterHandler()

{

Xil_Out8(XPAR_AXI_GPIO_1_BASEADDR+XGPIO_DATA_OFFSET,pos);//输出选择位码

Xil_Out8(XPAR_AXI_GPIO_1_BASEADDR+XGPIO_DATA2_OFFSET,segcode[i]);//输出此时i对应的段码

pos=pos>>1;//右移一位，从左到右扫描

i++;

if(i==8)//重置pos和i分别 为初始位码 以及 段码的起始位

{

i=0;

pos=0xff7f;

}

Xil_Out32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET,Xil_In32(XPAR_AXI_TIMER_0_BASEADDR+XTC_TCSR_OFFSET));

//将TCSR寄存器原来所存的值再次写入到TCSR寄存器中去，清除定时器的中断状态

}

//中断源为GPIO_2

void BTN_sw_seg()

{

char button=Xil_In8(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_DATA_OFFSET);

short cur_sw=Xil_In16(XPAR_AXI_GPIO_0_BASEADDR+XGPIO_DATA_OFFSET);

if(button==0x01)//BTNC

{

for(int i=0;i<8;i++)

{

for(int n=0;n<8;n++){

segcode[7-n]=segtable[(cur_sw>>n)&0x1];

}

}

}

else if(button==0x02)//BTNU

{

for(int i=0;i<4;i++){

for(int n=0;n<4;n++){

segcode[7-n]=segtable[(cur_sw>>(4*n))&0xf];

}

for(int n=4;n<8;n++){

segcode[7-n]=0xff;

}

}

}

else if((button==0x10))//BTND

{

int k=cur_sw;//不改变cur_sw

for(int j=0;j<5;j++){

switch(k%10)

{

case(0):segcode[7-j]=segtable[0];break;

case(1):segcode[7-j]=segtable[1];break;

case(2):segcode[7-j]=segtable[2];break;

case(3):segcode[7-j]=segtable[3];break;

case(4):segcode[7-j]=segtable[4];break;

case(5):segcode[7-j]=segtable[5];break;

case(6):segcode[7-j]=segtable[6];break;

case(7):segcode[7-j]=segtable[7];break;

case(8):segcode[7-j]=segtable[8];break;

case(9):segcode[7-j]=segtable[9];break;

}

k/=10;

}

for(int n=5;n<8;n++){

segcode[7-n]=0xff;

}

}

Xil_Out32(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_ISR_OFFSET,Xil_In32(XPAR_AXI_GPIO_2_BASEADDR+XGPIO_ISR_OFFSET));

//无论是是快速中断还是普通中断，都需要清除GPIO_2的中断状态，写ISR寄存器存的的1(申请中断状态)到ISR寄存器中去，清除GPIO_2的申请中断状态

}